Communications and Information Engineering Program 
Digital Signal Processing (CIE 442)

Fall 2025

Project 1 (Image Compression)

Project 1 Objective:

Compare the energy compaction properties of the Discrete Cosine Transform (DCT) and the Discrete Fourier Transform (DFT) for block-based image compression.

Tasks:

Each group shall include 2 members and are required to perform the following tasks:

Block-based transforms

- Load a gray scale image (e.g., camera man image). Resize to 256x256.

- Divide the image into non-overlapping 8×8 blocks (as in standard JPEG).

- Apply 2D DCT and DFT separately to each block.

- Test other block sizes (e.g., 4×4, 16×16) for comparison.

Coefficient reduction (compression)

- For each block, retain only the largest coefficients (by magnitude).

- Set the rest to zero.

- Experiment with different retention ratios (e.g., 10%, 25%, 50%).

Reconstruction

- Apply the inverse transforms to reconstruct the image for both DCT and DFT cases.

Analysis and comparison

- Display and save the original, DCT-reconstructed, and DFT-reconstructed images.

- Compute the Mean Squared Error (MSE) between the original and reconstructed images. Make sure to normalize the images before computing MSE.

- Discuss why DCT generally gives better quality for the same number of retained coefficients.

Deliverables

Your report (2–3 pages) must include:

- Short description of DCT and DFT principles (no formulas from AI tools — in your own words).

- Description of your experimental setup (block size, retention ratio, metrics).

- Plots/tables showing compression ratios and MSE comparisons.

- Discussion of results:

  - Why DCT shows higher energy compaction.

  - How block size affects performance.

  - What happens visually when too few coefficients are retained.

- All codes should be included in the report at the corresponding place.

Bonus (+2 points)

Implement a simple entropy coding stage:

- Run-Length Encode (RLE) consecutive zeros in the DCT coefficient matrix. Before the run length code you will need to transform each block from 2-D into 1-D vector using the following pattern

- Apply Huffman Coding to the RLE output to generate a bitstream.

- Implement the decoder to reconstruct the image from the encoded bits.

- Report the average bits per pixel (bpp) and compression ratio.

Rubric (Total = 10 Points)

| Criterion | Points | Description |
| --- | --- | --- |
| 1. Theoretical Understanding | 2 | Explains DCT vs DFT and Demonstrates understanding of why DCT achieves better energy compaction. |
| 2. Experimental Design and Coding | 3 | Clear setup (image, block size, retention ratio). Uses correct transforms and inverse transforms per block. Understands the code very well and its relation to theory. |
| 3. Results and Interpretation | 3 | Presents visual and numerical comparisons. Explains differences accurately and connects them to theory. |
| 4. Report Quality | 2 | Logical, well-structured report with clear conclusions and reflections. Shows understanding in their own words. |